🌐 System Overview Diagram – Noise

1 graph TD

2     A[Raw Audio: .wav/.m4a] --> B[Phase 1: Preprocessing]

3     B --> C[Phase 2: Segmentation]

4     C --> D[Phase 3: Feature Extraction]

5     D --> E[Phase 4: CNN Model]

6     E --> F[Phase 5: Aggregation]

7     F --> G[Final Decision: OK/NOT OK]

---

🛠️ Detailed Architecture Breakdown

PHASE 1: Signal Preprocessing (The "Cleaning" Lab)

* Format Standardization: Converts various formats to Mono WAV at 22,050 Hz.

* Noise Reduction: Uses the noisereduce algorithm to profile the first 1 second of "silence" and subtract

Bluetooth/Mic hiss from the entire signal.

* Spectral Filtering: A Butterworth Bandpass Filter (20Hz – 8,000Hz) is applied to isolate the frequency range where

BSR noises exist, discarding engine rumble and high-frequency electronic noise.

* Normalization: Peak amplitude scaling to ensure volume differences don't confuse the model.

PHASE 2: Segmentation (Sliding Window)

* Sliding Window: The 5-minute audio is sliced into 3-second chunks.

* Overlap (Hop): A 1-second step is used, creating an overlap. This ensures a rattle that happens at the edge of a

window is captured fully in the next one.

* Energy Gating: Chunks with RMS energy below a threshold are discarded to avoid training on "dead air."

PHASE 3: Feature Extraction (Audio → Image)

* Mel Spectrogram: The chunk is converted into a 2D representation.

* Y-axis: 128 Mel Scale frequency bins (optimized for human-like hearing).

* X-axis: ~130 Time frames.

* Value: Log-Decibel intensity.

* Output Shape: (128, 130, 1) — This essentially turns a sound clip into a grayscale image.

PHASE 4: Deep Learning Model (The "Brain")

* Architecture: ImprovedAudioCNN.

* Feature Learning: 3 Convolutional blocks (32 → 64 → 128 filters) learn to identify the "visual" signatures of

rattles (vertical spikes) or squeaks (horizontal streaks) in the spectrogram.

* Decision Layers: Two Fully Connected (Dense) layers with Dropout (0.5) translate learned patterns into a

probability score for two classes: [OK, NOT OK].

PHASE 5: Decision Aggregation (The "Verdict")

* Voting Mechanism: Since the model predicts on 3-second chunks, a 5-minute file generates ~280 predictions.

* Logic:

* If any chunk is flagged as "NOT OK" with high confidence, the entire vehicle is flagged.

* Confidence Scoring: The mean probability of all "NOT OK" chunks provides a severity score.

---

📊 Technical Stack

┌──────────────────┬───────────────────────────────────────────┐

│ Component        │ Technology                                │

├──────────────────┼───────────────────────────────────────────┤

│ Language         │ Python 3.11+                              │

│ Deep Learning    │ PyTorch / TorchAudio                      │

│ Audio Processing │ Librosa / SciPy                           │

│ Noise Reduction  │ Noisereduce (Stationary masking)          │

│ Metrics          │ Scikit-Learn (Confusion Matrix, F1-Score) │

└──────────────────┴───────────────────────────────────────────┘

🚀 Implementation Status in your Workspace

I have already implemented this flow across the following files in NoiseClassification/:

1. `preprocessing.py`: Handles cleaning and filtering.

2. `segmentation.py`: Handles window slicing.

3. `features.py`: Converts audio to Mel Spectrograms.

4. `model.py`: Contains the CNN architecture.

5. `train_v2.py`: Coordinates the end-to-end training.

6. `inference_v2.py`: Coordinates the file-level prediction logic.